APPENDIX No. 1. TECHNICAL SPECIFICATION

Functional and technical parameters of the granite aggregate processing plant:

| Item No. | Functional and technical parameters | Parameter values |
| --- | --- | --- |
| 1 | 2 | 3 |
|  | Processing plant capacity and products | The granite aggregate processing plant operates efficiently at a capacity of 310 t/h when producing 0/22 mm material and at 230 t/h when producing 0/16 material. The plant is designed to produce the following fractions: 0/2, 2/5, 5/8, 8/11, 11/16 and 16/22. |
|  | FLSmidth Raptor 450 cone crusher | - capacity: 190–650 t/h; - power supply: 400/690 V, 50 Hz; - maximum size of material to be crushed: 270 mm; - minimum size of material to be crushed: 5 mm; - electric motor power: 320 kW; - electric motor speed: 1,400 rpm; - drive speed: 690 rpm; - gap between the cones: 10–50 mm; - hydraulic oil tank capacity: 200 L; - lubrication tank capacity: 600 L; - automatic lubrication and hydraulic system; - protection against uncrushable material; - crusher control system; - crusher weight: 33,000 kg; - crusher height: 3,000 mm |
|  | 25 m³ hopper with 100 × 300 feeder | - hopper capacity: 25 m³; - vibrating feeder nominal size: 1,000 × 3,000 mm; - vibrating feeder power: 2 × 2.7 kW; - supporting metal structure, galvanised |
|  | Three-deck vibrating screen 240 × 600/3 | - motor power: 2 × 22 kW - vibrating screen nominal size: 2,400 × 6,000 mm; - metal screen mesh; - dust cover, easily removable; - electric brake; - galvanised supporting metal structure with maintenance platform; - armoured metal chutes; |
| - 3. | Two-deck vibrating screen 240 × 600/2 | - motor power: 30 kW; - vibrating screen nominal size: 2,400 × 6,000 mm; - metal screen mesh; - dust cover, easily removable; - electric brake; - galvanised supporting metal structure with maintenance platform; - armoured metal chutes; |
|  | Two-deck vibrating screen 240 × 700/2 | - motor power: 37 kW; - vibrating screen nominal size: 2,400 × 7,000 mm; - metal screen mesh; - dust cover, easily removable; - electric brake; - galvanised supporting metal structure with maintenance platform; - armoured metal chutes; |
|  | Stationary conveyor T-1 | - Covered conveyor with galvanised structure; - Maintenance walkway on one side; - Length: 10 m; - Belt width: 1 000 mm; - Angle: 7°; - Capacity: 320 t/h; - Power: 5,5 kW; - Belt speed: 1,27 m/s |
|  | Stationary conveyor T-2 | - Covered conveyor with galvanised structure; - Maintenance walkways on both sides; - Length: 31 m; - Belt width: 1 000 mm; - Angle: 14°; - Capacity: 345 t/h; - Power: 2 × 11,0 kW; - Belt speed: 1,18 m/s |
|  | Stationary conveyor T-3 | - Covered conveyor with galvanised structure; - Maintenance walkways on both sides; - Length: 55 m; - Belt width: 650 mm; - Angle: 18/3°; - Capacity: 96 t/h; - Power: 2 × 7,5 kW; - Belt speed: 1,14 m/s |
|  | Stationary conveyor T-4 | - Covered conveyor with galvanised structure; - Maintenance walkway on one side; - Length: 8 m; - Belt width: 650 mm; - Angle: 5°; - Capacity: 96 t/h; - Power: 2,2 kW; - Belt speed: 1,10 m/s |
|  | Slewing conveyor T-5 | - Covered conveyor with galvanised structure; - Maintenance walkway on one side; - Length: 25 m; - Belt width: 800 mm; - Angle: 18°; - Capacity: 147 t/h; - Power: 2 × 5,5 kW; - Slewing mechanism: 0,75 kW; - Belt speed: 1,34 m/s |
|  | Stationary conveyor T-6 | - Covered conveyor with galvanised structure; - Length: 7 m; - Belt width: 650 mm; - Angle: 15°; - Capacity: 69 t/h; - Power: 3 kW; - Belt speed: 1,15 m/s |
|  | Slewing conveyor (T-7, T-15), 2 units | - Covered conveyor with galvanised structure; - Maintenance walkway on one side; - Length: 25 m; - Belt width: 650 mm; - Angle: 18°; - Capacity: 63–69 t/h; - Power: 7,5 kW; - Slewing mechanism: 0,75 kW; - Belt speed: 1,34 m/s |
|  | Stationary conveyor T-8 | - Covered conveyor with galvanised structure; - Length: 10 m; - Belt width: 1 000 mm; - Angle: 15°; - Capacity: 230 t/h; - Power: 7,5 kW; - Belt speed: 1,14 m/s |
|  | Stationary conveyor T-9 | - Covered conveyor with galvanised structure; - Maintenance walkways on both sides; - Length: 57 m; - Belt width: 1 000 mm; - Angle: 18/0°; - Capacity: 230 t/h; - Power: 2 × 15 kW; - Belt speed: 1,18 m/s |
|  | Stationary conveyor T-10 | - Covered conveyor with galvanised structure; - Maintenance walkways on both sides; - Length: 40 m; - Belt width: 800 mm; - Angle: 18/0°; - Capacity: 142 t/h; - Power: 2 × 9,2 kW; - Belt speed: 1,16 m/s |
|  | Stationary conveyor T-11 | - Covered conveyor with galvanised structure; - Length: 6 m; - Belt width: 650 mm; - Angle: 13°; - Capacity: 50 t/h; - Power: 3 kW; - Belt speed: 1,15 m/s |
|  | Slewing conveyor (T-12, T-16, T-19), 3 units | - Covered conveyor with galvanised structure; - Maintenance walkway on one side; - Length: 25 m; - Belt width: 650 mm; - Angle: 18°; - Capacity: 38–50 t/h; - Power: 7,5 kW; - Slewing mechanism: 0,75 kW; - Belt speed: 1,34 m/s |
|  | Stationary conveyor (T-13, T-17), 2 units | - Covered conveyor with galvanised structure; - Length: 10 m; - Belt width: 650 mm; - Angle: 15°; - Capacity: 38 t/h; - Power: 3 kW; - Belt speed: 1,15 m/s |
|  | Stationary conveyor T-14 | - Covered conveyor with galvanised structure; - Length: 27 m; - Belt width: 650 mm; - Angle: 7°; - Capacity: 38 t/h; - Power: 5,5 kW; - Belt speed: 1,08 m/s |
|  | Stationary conveyor T-18 | - Covered conveyor with galvanised structure; - Length: 20 m; - Belt width: 650 mm; - Angle: 9°; - Capacity: 38 t/h; - Power: 4 kW; - Belt speed: 1,07 m/s |
|  | Dust-suppression water spraying system | - designed for and installed on the granite aggregate processing plant; - installed on the movable conveyors at the discharge points into the stockpiles (6 points); - the dust-suppression water spraying system is integrated with the control system of the entire processing plant. |